iLife Editor 3.10 Instructions

(Version: 3.10 Official Version)

CONTENTS

1、Start the main interface of the software：	1

2、Room Home Page Setting	2

2.1、Room information box	2

2.2、When you choosing the room name	3

2.3、Devices List Box	3

3、Home page devices display setting	5

3.1、TV	5

3.2、Lighting	10

3.3、Air Conditioning	12

3.4，Scene	14

3.5、DVD	17

3.7、Background Music	18

3.8、Media	19

3.9、PA	20

3.10、Monitoring	21

3.11、Security	22

3.12、Blue-ray	23

3.13、Projector	24

3.14、Acoustics	25

3.15、DIY	26

3.16、Matrix	26

3.17、Karaoke	27

3.18、Geothermal System	28

3.19、Smart Meter	29

3.20、DiiVA	30

3.21、Home Theater	31

3.22、Power Setting	32

4、Hotel Control	34

5、Access Control Setting	35

6、Temperature Setting	36

7、Local Setting	37

8、Upload File	37

9、Help	38

# Start the main interface of the software：

Start interface：

Project management page：You can create a new project, delete or rename a project, browse the database folder of the project;

-You can double click on the project name and enter into the main interface of project selection as below：

1)Home Page：Software startup page, pictured above

2)The room list box：Show all the room name;

The room information box：Click on a room in the room list and it will show the display name of the room and its icons;

3)Room list on the right side of the software：you can click on “add” to add new room to the room and its display icon.

4)Device list on the right side of the software：double click on any device to add it to a specific room.

5)DIY Setting：Click to open the DIY setting page（Disabled）

6)Local Setting：Click to open the local settings page

7)Alarm：Alarm setting（Disabled）

8)Access Control：Click to open the access control page

9)Voice Control：Click to open the voice control page（Disabled）

10)Temperature：Click to open a temperature display settings page

# 2、Room Home Page Settings

2.1、Room information box：

Add new room：Click the “Add button” to add a new room quickly ,

Delete room：Choose the room name, click the delete button to delete the room and related data.

2.2、modify  the room information：

Rename room：first select a room and write the new name in the input box directly to modify , do not need to press the Save；

Modify the room Icon：Choose the room, click on the “room icon” on the bottom left corner ,so  the middle region will display a list of icons, choose one to Change Icon;

Delete Button：don’t delete the room button unless you get rid of that room, because all related data will be deleted too.

2.3、Devices List Box：

Add room device：Select a room, on the device display frame, double-click on one device, it  will be added to the room, and then displayed on the display area：

-Right click the device name, it will show two options:-  delete and property,

-the property settings window as shown below：

Rename device：Select the device that you would like to rename , click the Save button

Device Order：Click the up or down button to sort, sort order in the iLife display consistency.

# 3、Home page devices display settings

3.1、TV：

Ilife allows you to control your TV easily, all you need is to use IR emitter or any other HDL product which can transmit an IR code,

Learn the TV remote controller buttons on HDL IR learner and fill each buttons IR UV switch in Ilife TV tab and you will be able to control your TV.

- Set-top Box

Click on the TV device, the default display is set-top box page.

Single：To modify the selected row;

Multi-line：Modify a multi-line at the same time.

Auto Accumulate：To modify a numeric value, the column all numeric values will become progressively (increment value 1);

Save Button：don’t forget to SAVE after you make any modification.

Multi-line Modify：Click the Set-top Box button，to modify all the keys；

Single Modify：Click on a designated button, to modify its target.

iLife Editor, The copy and Paste function ：

a、Right click on the row that you want to copy, and paste it on the destination.

- Program Collection

You can add some favorite TV channels here, so it will be easy for the users to switch to that assigned channels,

Add program：Enter a name, click the Add button to add new program

Show checkbox：Tick checkbox will display name and picture in iLife, or only show picture.

Delete program：Choose the program name, click the delete button;

Program Icon modify：Choose the program name, and then click Image button to open the icon list box, double-click the icon to change.

Import Picture：Importing a new icon to the display frame, picture format is PNG, best pixel is 242X199;

Remove Picture：If remove the icon, there won’t be displayed any icon:

Category Button：Classified management of all programs, as the following diagram：

List of all programs：All program, support multiple choice

Cancel Button：Clear all program selection;

>>Button：The selected programs will append to the chosen category;

<<Button：removes the selected program From the current category ;

Clear Button：Empty the current category including program;

3)Satellite TV

Save Button：Save changes of the target rows;

Delete Button：Remove all the rows of the selected satellite TV button;

Delay Time：Between rows of the interval time，the default is millisecond time unit;

3.2、Lighting

Add new lighting loop：Click the blank area of lighting list, input a name to the textbox, then click add button to add a new lighting loop;

Modify lighting loop：Select the lighting loop so you can modify its targets. click the Save button to save the changes;

Delete lighting loop：Select the lighting loop, click the Delete button to delete its data;

Add new lighting scene： Click on the middle blank button, enter a name, click the Save button. Add a targets to increase the lighting scene target rows;

Modify lighting scene: First select the light scene name button and you can modify and save;

Delete lighting scene: First select the middle light scene name button, click the delete button to delete;

3.3、Air Conditioning

Name：Air conditioning name

Type selection：Module Control、IR Normal Control、IR Central Control

Subnet No.：The subnet ID of module control;

Device No.：The device ID of module control;

The default is the module control type, pictured above, if you choose the IR Normal control and IR central control, as shown below：

Temp Type：This is the temperature type, if you choose ℃, the range is 0 to 30 DEG; and if choose ℉,the range is 32 to 86 degrees;

Display checkbox: If tick checkbox mean iLife has cold button in the air conditioning control page;

Save Button：after any modification you must press the Save button to save;

Delete Button：Click to delete the current selection of air conditioning;

Auto，High-wind，Medium-wind，Low-wind：Optional air conditioning mode represents the wind speed function;

Set Button：Temperature type, temperature range, wind speed change,

3.4，Scene

You add some scenario here as you like, this eases the use of this application, for example you don’t need turn on/off light1, light2,curtian1,curtain2,AC1 one by one when it’s in one room, you can just create a scene and turn on/off all of them by one click

Add new scene：Enter a name, select the scene type, such as a single on / off or momentary, and then click the Add button to add the new scene;

Modify scene：Select the scene name, to modify;

Delete scene：Select the scene name, to delete;

Target1：When the scene type is single on / off, one can use only the Target1;

Target2：When the scene type is momentary, both can use the Target1 and Target2;Momentary scene means iLife scene button down and up can send different target rows;

Modify Scene Icon：Select the scene name，click the Image button, picture selection box appears, and then double-click the icon to change;

When the icons list appear, choose the scene that you want to change, and then double-click on the icon directly to modify;

Scene link setting：Right click the scene list box, select the link setting;

Scene link setting：As following diagram:

Delete scene link：Click the delete button, will delete the selected scene link page value;

Clear all scene link：Click the Clear button, will empty all scene link page value;

Cancel Button：Click the Cancel button to cancel all the selected scene of the list box;

Set scene link：Click the Setting button, the currently selected scene link value will be set to the link device drop-down box to the current device page;

3.5、DVD

Multi-line Modify：Press the blank area, you can modify all button target;;

Single Modify：Press one button,you can single modify button targets;

3.6、Curtain

Add new curtain：Enter a name, click the Add button to add the curtains and it will automatically generate three target rows;

Modify curtain：Select the curtain name，then you can add the target curtain

Delete curtain：Select the curtain name，then you can delete;

3.7、Background Music

Add background music control：Click on the top blank buttons, enter a name, and save

Modify background music control：Select background music name，you can modify and delete;

Module Type：there are 2 types, OLD= for HDL old music box hardware,  NEW=HDL new hardware music box, please choose the corresponding option

Enable Z-Audio Playlist：Tick this option first so you can drag the PLS file to the Z-Audio list box;

Z-Audio List：Select the list song category, right-click to delete.

3.8、Media

Its used to control MediaServer which is developed by HDL, MediaServer can voice control music or film play and picture slide browsing;

Multi-line Modify：Press the blank area，you can modify all button values;

Single Modify：Press one button，you can single modify button values;

3.9、PA

Audio power amplifier device;

Multi-line Modify：Press the blank area， you can unified modify all button values;

Single Modify：Press one button，you can single modify button values;

3.10、Monitoring

Its used to monitor any IP camera from your Ipad , Iphone and any android device

Add new camera：Click the top blank button, enter a name, and then Save to add a new camera;

Modify camera：Select the camera name，you can modify and delete;

Supported Types：Hikvision、WINDAY、Network URL…….

Local、Remote：Display when you choose the WINDAY type，Local mean use LAN access camera, Remote using the Internet to access the camera;

IP Address：camera IP address

Port：camera port number

User Name： the camera's login name

Password： the camera's password

Channel No.：Each channel number means a camera.

3.11、Security

Security Name：Name of security device;

Subnet No.：The subnet ID of security device;

Device No.：The device ID of security device;

Zone No.：A security module has 8 default zone number;

Zone Name： The name of the corresponding the zone number;

Top Save button：Modify security

Top Delete button：Press will delete all data of the security;

Type：Six kinds of arming setting in teach zone ,

Status：Configuration information for sensor

3.12、Blue-ray

Multi-line Modify：Press the blank area， you can modify all button targets;

Single Modify：Press one button，you can modify single button target;

3.13、Projector

Multi-line Modify：Press the blank area，so u can modify all button targets

Single Modify：Press one button，you can modify a single button target.

3.14、Acoustics

Add First layer Button：Click the top blank button, enter a name, click the Add button to add first layer button;

Delete First layer Button：Select first layer button，you can modify and delete the first layer button;

Second layer Button：Select first layer button at first，you can modify the second layer button;

3.16、Matrix

Add First layer Button：Click on the top row blank button, enter the name, click the Add button to add a First layer Button;

Modify First layer Button：Select on the top row button，you can modify the name and targets;

Delete First layer Button：Select first layer button，you can delete it and the second layer button will be deleted also;

Add Second layer Button：You must add the first layer button first and then choose the second blank button, enter the name，click the Add button to add a secondary button;

Modify Second layer Button：You must select a first layer button, then you  can choose the second button, click the Save button to modify;

Delete Second layer Button：You must select a first layer button, then can choose the secondary button, click the Delete button to delete;

Image Button：Press to open picture list，then double click picture to save the changes;

3.17、Karaoke

Multi-line Modify：Press the blank area， you can modify all button targets;

Single Modify：Press one button，you can single modify button targets

3.18、Geothermal System

This device is used to control HDL floor heating module, and controlled through DLP panel;

Add geothermal module control：Click the blank button, enter the name, click the Save button to add the geothermal module control;

Modify geothermal module control：Select the button name，you can modify and delete the data;

Subnet No.：the subnet ID of geothermal module control

Device No.：the device ID of geothermal module control

Channel No.：the channel number of geothermal module control，

Note:- 0 cannot be used as a channel number

3.19、Smart Meter

Add smart meter control：Click the blank button, enter the name, click the Save button to add smart meter control;

Modify smart meter control：Select the button name，you can modify and delete the data;

Subnet No.：the subnet ID of smart meter control

Device No.：the device ID of smart meter control

Type：single phase and three-phase type

3.20、DiiVA

Subnet No.：the subnet ID of DiiVA

Device No.：the device ID of DiiVA

Save Button：The subnet ID, device ID, zone number, zone name，source number，source name, source icon must click the Save button to save changes;

Add Button of Zone：Enter the number and zone name directly and it will be added successfully, do not need to click the Save button;

Delete Button of Zone：Select a item of zone list, press the Delete button directly delete，do not need to click Save button;

Add Button of Source：Enter the number and source name, and select the icon, click Add direct success does not need to click the Save button;

Delete Button of Source：Select an item from source list, press the Delete button directly delete，

3.21、Home Theater

In order to facilitate the selection of frequently used device;

Select the type of device：Has been added to the list of all the equipment in the room;

Add Button：Add the selected device to a home theater management;

Delete Button： Remove the device from the device type list, not really to delete the device;

Up and Down Button：Click Sort upward or downward to the home theater list of device, and it will display on ilife as you sort it.

3.22、Power Setting

Add Power Control：Enter a name， click the Add button to add power control

Modify Power Control：Select the power control name，so you can modify it;

Delete Power Control：Select the power control name，you can delete it;

Power Control Icon：Click on the Image button, open the picture list box, click a different picture so you can directly modify it;

Icon Status：Click the open or closed status, will reload the state image display;

# 4、Hotel Control

Right click on the room, select hotel management, there will be a pop-up window as above;

IP：The room IP address, judging the IP input is valid;

Port：The default port 6006

MAC： MAC physical address      Password：Default 12

# 5、Access Control Setting

You can add here many camera to monitor, and also can create some scenes.

Networks URL：cameras type

WINDAY：cameras type

Hikvision：cameras type

Add Access Scene： Click the blank button, enter a name, click the Add button to add scene;

Delete Access Scene： Select access scene button，click Delete button to delete

Note: access all modifications must click the Save button to save changes;

# 6、Temperature Setting

Subnet No. of Indoor：Subnet ID of Indoor temperature

Device No. of Indoor：Device ID of Indoor temperature

Channel No. of Indoor：Channel number of Indoor temperature

Temperature checkbox of Indoor：Tick checkbox indoor temperature will display in iLife;

Subnet No. of Outdoor：Subnet ID of Outdoor temperature

Device No. of Outdoor：Device ID of Outdoor temperature

Channel No. of Outdoor：Channel number of Outdoor temperature

Temperature checkbox of Outdoor：Tick checkbox outdoor temperature will display in iLife;

Save TXT Button：Modify the relevant value and click the button to generate a new TXT file;

# 7、Local Setting

Security Lock Password：IPAD screen saver password

PC Server IP address：An HDL MediaServer media software the computer address,avoid if a LAN in if you open more than one MediaServer, can simultaneously control a plurality of MediaServer; if the address is specified only on the specified address where the MediaServer control;

# 8、Upload File

Open the file menu select upload file function：

UpLoad Button：One click upload related configuration file to IPAD;

Connecting device type：Support upload configuration files to the Apple and Android devices;

Note：iLife Editor Computer and equipment must be the same network, computer is best wired access network, if the computer is the WIFI access network is likely to occur can not upload the situation;

How to download the IPAD software, please see the IPAD tutorial

# 9、Help 113.108.195.230

Soft Document：iLife Editor Help manual

Check new version：Online check software version, if not the latest version can be online updated directly;

Update Log：The update log viewer software upgrade

Contact Us：Contact information